# Error Codes

## 0 of 0 Tuners in Use

- Meaning

- Receiver is in a fail state

- Cause

- Cable issues, make sure receivers are activated, hard drive failure

- Recommended Resolution

- Check the cable as it could be a MoCa issue

- If not, reboot receiver using front panel Reset button or by unplugging power cord

- Wait for the receiver to reboot and then verify tuners are available

## 002

- Meaning

  - Partial Signal Loss

- Cause

  - Signal loss due to inclement weather

  - Misaligned or damaged dish antenna

  - Failure of component

  - Improperly installed component in signal path

- Recommended Resolution

  - Using a signal meter, check signal at the receiver location and work back toward the dish antenna

  - Check signal at each connection point

  - Examine coax cable, connectors and components for proper installation and approved parts

  - Correct signal issue at dish antenna

## 004

- Meaning

  - Channel Signal Loss

- Cause

  - This error will appear if there is a problem with the multi-dish switch

  - This can also be caused by using incompatible or malfunctioning equipment between the receiver and the switch

- Recommended Resolution

  - Run a check-switch test on the receiver to verify connectivity to the switch/LNBF and availability of satellites

  - Examine coax cable, connectors and components for proper installation and approved parts

## 005

- Meaning

  - Programming Not Authorized

- Cause

  - Current channel is not authorized on the receiver

- Recommended Resolution

  - Verify the receiver is active on the customer account

  - Confirm the receiver and smart card numbers on the account match the physical receivers

  - Verify desired programming is active by checking OFSC or by contacting the call center

## 010

- Meaning

  - Receiver unable to connect to Internet using internal Wi-Fi

- Cause

  - Incorrect settings or a problem with the Internet connection

- Recommended Resolution

  - Reboot receiver using front panel reset button or by unplugging power cord

  - On the receiver access MENU > SETTINGS > INTERNET > WIRELESS SETUP and verify the desired WiFi network is selected and the password has been entered correctly

  - If this doesn't work, adding a HIC can be beneficial or reset the WiFi settings by pushing INFO > RECAL when in the network diagnosis screen

## 013

- Meaning

  - Authorization

- Cause

  - Programming isn't authorized on the receiver

- Recommended Resolution

  - Verify the equipment is activated in OFS

  - If the receiver is active in OFS contact DASH to verify receiver/smartcard number and programming

## 015

- Meaning

  - Complete Signal Loss

- Cause

  - Signal loss due to inclement weather

  - Misaligned or damaged dish antenna

  - Obstruction blocking satellite signal

  - Failure of component

  - Improperly installed component in signal path

- Recommended Resolution

  - Using a signal meter, check signal at the receiver location and work back toward the dish antenna

  - Check signal at each connection point

  - Examine coax cable, connectors and components for proper installation and approved parts

  - Verify dish antenna is installed and aligned in accordance with installation standards

## 015D

- Meaning

  - Weather Related Signal Loss

- Cause

  - Receiver is not receiving satellite signal but can still detect the LNBF or switch in the check switch result

- Recommended Resolution

  - Verify satellite signal by running a check-switch test on the receiver and or checking signal with a satellite meter

  - This message is common during a weather related signal event

  - Remove snow from dish antenna when possible or wait for weather event to pass

## 019

- Meaning

  - Authorization

- Cause

  - This message will display if the smartcard is not inserted correctly, the receiver's software is outdated, or the card or receiver's card reader is defective

- Recommended Resolution

  - Confirm the software on the receiver is current

  - Remove the smartcard and check for any damage to the card

  - Reinsert the card into the reader. If this does not resolve the issue, replace the receiver

## 021

- Meaning

  - Authorization

- Cause

  - The smartcard was removed from the receiver, or if there is a problem with the smartcard reader of the receiver

- Recommended Resolution

  - Confirm the software on the receiver is current

  - Remove the smartcard and check for any damage to the card

  - Reinsert the card into the reader. If this does not resolve the issue, replace the receiver

## 024

- Meaning

  - Miscellaneous

- Cause

  - A feature that relies on guide information such as favorites, timers, DVR events etc. is being used and the receiver does not have access to the hard drive

- Recommended Resolution

  - Reset the receiver and attempt a manual guide update

    - If there is error 311 occuring on the receiver as well, the hard drive failure is causing the error

## 031

- Meaning

  - Miscellaneous

- Cause

  - Ordering a program that is not available for purchase

  - If the channel is listed as "Off Air" in the guide, this attention message appears when the channel is not scheduled to show programming

- Recommended Resolution

  - Educate the customer that this channel is not currently showing any content so no PPV events can be purchased

## 058

- Meaning

  - Checkswitch in Progress

- Cause

  - The receiver is performing a checkswitch

- Recommended Resolution

  - Allow the checkswitch to finish and review the results

## 120

- Meaning

  - Outside Of Viewing Area

- Cause

  - This message is displayed when a customer accesses a Spot Beam channel after moving their receiver to a new location that is outside of the coverage area

  - If they then go directly to a non-DMA channel, they may also get this message

- Recommended Resolution

  - Verify satellite signal by running a check-switch test on the receiver and or checking signal with a satellite meter

  - Explain to the customer that local channels are restricted to certain geographic areas

    - This message is common when a customer moves from one location to another and has not yet updated locals on the account. If this appointment is a DISH mover, contact DASH to ensure the correct locals are on the work order and have the agent send an authorization hit to begin receiving the new locals

  - Confirming the locals are correct on the account is a good best practice on all work orders when this error message appears

## 122

- Meaning

  - Smartcard Communication Error

- Cause

  - An internal communication failure has been detected

- Recommended Resolution

  - Press "select" on your Remote Control to reset your receiver

  - If resetting the receiver does not work, you'll need to replace it

## 145

- Meaning

  - Connectivity

- Cause

  - This message appears when the customer accesses On Demand without an Internet connection

- Recommended Resolution

  - Connect the receiver to the internet using a direct ethernet connection, WiFi or HIC

## 303

- Meaning

  - Hard Drive

- Cause

  - The normal initial software download

  - The receiver will update the screen with instructions as it completes this process

  - If the receiver is disturbed, the download process may have to start over again

- Recommended Resolution

  - Allow the receiver to finish downloading software

## 311

- Meaning

  - Hard Drive Failure

- Cause

  - This message appears when the receiver is unable to mount the hard drive (internally) or the receiver detects a fatal failure

- Recommended Resolution

  - Confirm the outlet providing power is wired correctly

  - To prevent this error ensure the receiver is in a well ventilated area and do not move the receiver while it is plugged in

  - Replace the receiver

## 349

- Meaning

  - Smartcard Incompatible

- Cause

  - The smartcard inserted could be a past generation that is not compatible or damaged

- Recommended Resolution

  - Check the smartcard number associated with the receiver on the account and confirm whether it matches the number on the smartcard in the slot on the left-hand side of the receiver

  - If the smartcard does not match the receiver number, remove the smartcard from the slot and hold the power button down for 5 seconds until it resets

  - If the smartcard matches the receiver number, remove the smartcard from the slot and hold the power button down for 5 seconds until it resets, then wait for the receiver to come back online and reinsert the smartcard

  - If the error is still occurring, replace the receiver

## 371

- Meaning

  - Programming Not Authorized

- Cause

  - This message appears when tuning to a channel that is not currently a part of the programming authorized on the account

- Recommended Resolution

  - Validate that the customer does not subscribe to this channel

  - Educate the customer on their channel package and how to upgrade if they would like to begin receiving this channel

## 457

- Meaning

  - The remote has reached the pairing limit

- Cause

  - This message appears when the Dish remote has been paired with 4 or more TVs in a 24 hour period

- Recommended Resolution

  - Wait 24 hours then pair the remote

  - If you need to pair the remote to the TV today, the only resolution is to get a new remote

## 514a

- Meaning

  - Inactivity Standby

- Cause

  - The receiver is about to go into standby mode due to inactivty

- Recommended Resolution

  - Inactivity standby allows the receiver to take essential software updates

  - The length of time can be increased/decreased based on the customer's preference in the settings menu

  - Turning the inactivity standby off completely is not recommended

## 535

- Meaning

  - Acquiring Signal

- Cause

  - This message appears when the receiver has lost signal and is in the process of reacquiring

  - This may occur after a reset

- Recommended Resolution

  - Allow the progress to complete to resume viewing

  - If signal is not restored then perform signal path troubleshooting

  - Verify the DISH antenna is installed and aligned in accordance with installation standards

## 536

- Meaning

  - Local Channels Interrupted

- Cause

  - Local channels temporary outage

- Recommended Resolution

  - Perform all standard signal troubleshooting to ensure installation is functioning normally

  - Validate that there is an outage by checking Compass real time updates or calling DASH

  - Notify the customer that there is an issue outside of their dish installation and service will return after the issue is corrected

## 833

- Meaning

  - Signal

- Cause

  - Invalid or missing installation summary is detected, a check switch is run during a period of low signal or in a faulty installation, a check switch was not properly run during initial installation

- Recommended Resolution

  - Let the check-switch test complete and follow normal satellite signal troubleshooting including using a satellite meter to verify signal is adequate

  - Inspect coax cable, connections, components and dish antenna for proper installation

## 835

- Meaning

  - Signal

- Cause

  - The Auto Checkswitch completes and encounters an error

- Recommended Resolution

  - Verify orbitals are being received using a satellite meter

  - If orbitals are missing, check LOS and cable path

  - Run a checkswitch once any issues have been addressed

## 836

- Meaning

  - Equipment

- Cause

  - This message appears after a check-switch test when the receiver detects fewer satellites than before

  - This message is common when one satellite that was previously available is not detected

    - This can occur when a weather event, obstruction temporarily impact signal or a change in orbitals used at the dish (Arc Swap)

- Recommended Resolution

  - Perform signal path troubleshooting

  - Verify dish antenna is installed and aligned in accordance with installation standards

  - If you are expecting fewer orbitals than originally were on the STB, save the results

## 890

- Meaning

  - High-bandwidth Digital Content Protection (HDCP)

- Cause

  - This message appears when selecting an HD Premium Channel, PPV, VOD, or DishONLINE event to order, AND the customer’s TV is NOT HDCP-capable AND they are connected with an HDMI or DVI cable

- Recommended Resolution

  - This is typically caused by out of date firmware on the customer's TV

  - Advise the customer to have the TV firmware updated

  - This can usually be accomplished through the manufacturers website which will have any available firmware updates listed by model number

  - Try a different TV in the customer's home to ensure the receiver is displaying the content

## 908

- Meaning

  - Remote

- Cause

  - The remote is not in SAT mode

- Recommended Resolution

  - Press the SAT button on the remote to remove this pop up

## 910

- Meaning

  - Connectivity

- Cause

  - When viewing a program at the same time as it is downloading, this message will appear when the playback catches up with the download progress

- Recommended Resolution

  - Allow the receiver time to build up buffer space again

## 915

- Meaning

  - Account

- Cause

  - This message is prompted when a customer tries to order a PPV using the remote

- Recommended Resolution

  - Connect the receiver to the internet using a direct ethernet connection, WiFi or HIC

  - If internet is not available the customer can order PPV events through the DISH customer service phone number

## 981

- Meaning

  - Boot Recovery

- Cause

  - The receiver has detected a failure with the internal hard drive. To prevent this error ensure the receiver is in a well ventilated area and do not move the receiver while it is plugged in.

- Recommended Resolution

  - Allow the receiver to finish the boot recovery process

    - If the receiver doesn't progress, replace the receiver

  - Run STBH checking for hard drive failure

  - Check electrical with receptacle tester making sure power is okay

  - Check cable connections

  - Replace receiver

## 1158

- Meaning

  - Your Wireless Joey is not connected to a Wireless Joey Access Point

- Cause

  - The Wireless Joey and Access Point (AP) have lost the connection due to an installation issue such as a disconnected AP

- Recommended Resolution

  - Unplug AP and Wireless Joey then power on to re-establish connections. Follow on screen instructions to reconnect the Wireless Joey to the AP

  - Reposition AP and Wireless Joey closer together and away from other electronic devices such as routers

  - Verify the Wireless Joey is active on the account and linked to the Hopper

## 1228/875

- Meaning

  - Connectivity

- Cause

  - This message appears when the customer resets the network in SETTINGS > DIAGNOSTICS > NETWORK

  - Message 875 is the equivalent in other receivers

  - "Hopper is configuring the network to provide Joeys with connectivity. This will briefly disrupt the use of the Joeys and other network access."

- Recommended Resolution

  - Allow the receiver to finish configuring the network

## 1243a

- Meaning

  - Connectivity

- Cause

  - This message appears when the customer accesses On Demand or another feature that requires Internet connectivity when the receiver is not connected to the Internet

- Recommended Resolution

  - Connect the receiver to the internet using a direct ethernet connection, WiFi or HIC

## 1259

- Meaning

  - Connectivity

- Cause

  - This pop-up occurs when the receiver is unable to join a Broadband network

- Recommended Resolution

  - Use a different connection method, Ethernet, WiFi or HIC

## 1303

- Meaning

  - Looking for server/Hopper

- Cause

  - Coax cable to Joey is damaged or incorrectly connected or Joey is not authorized on account

- Recommended Resolution

  - Inspect the signal path starting at the Joey and working back toward the Hub/Node

  - Check all connections and coax cable to verify correct installation of connectors and components

  - Inspect coax cable for damage and replace as necessary

  - Verify the Hopper is correctly connected to Hub/Node

  - For multiple Joey installations, the Joeys can be swapped to test if a particular location is the problem or if the problem follows the Joey

## 1304

- Meaning

  - Broadband Connection Lost

- Cause

  - Poor MoCA Signal

- Recommended Resolution

  - Perform a front panel reset on the Joey and Hopper

  - Ensure the Joey is linked to the Hopper in SETTINGS > WHOLE HOME

  - Inspect the signal path starting at the Joey and working back toward the Hub/Node

  - Some routers can interfere with MoCA signal, to solve this attempt a different broadband connectivity method including installation of a HIC

## 1305

- Meaning

  - Broadband connection lost

- Cause

  - Hopper is disconnected from internet or incorrectly connected. Router is not compatible with Hopper system

- Recommended Resolution

  - Attempt a connectivity method that is different from the current method

  - If a router is not allowing Joeys to connect to a Hopper then the Hopper Internet Connector (HIC) is advised

## 1309

- Meaning

  - Joey connection to the Hopper has been lost

- Cause

  - Cabling issues

  - Installation configuration issues

  - Network issues

- Recommended Resolutions

  - Check all cable runs for issues, failed components

  - Ensure Client connections are correct

  - Enable Wireless-Ethernet Bridging (WET)

    - IF the router has a setting for WET (many routers may not), this should be ENABLED

  - Increase DHCP Lease Time

    - DHCP Lease Time in the router settings should be set for 24 hours or longer

      - This is often set in minutes, with 1440 minutes equaling 24 hours

  - Disable DHCP when 2 routers are on the same network

    - If there are 2 routers on the network (for instance, the customer has a gateway provided by their ISP and also a mesh Wi-Fi system), disable DHCP on the first router

      - Bridge Mode should also be turned off for the router that you are disabling DHCP on

  - Create a 2nd Subnet

    - Install a 2nd router to create a separate subnet with only the Hopper system connected to it

    - This process is outlined in Receiver Broadband Connectivity

- Best practices

  - Have the customer log in to the router and navigate/change settings themselves

  - Perform a web search for "default username and password" and "router login url" and include the ISP and model of the customer's router in the search

    - This will tell you which IP address/URL to navigate to in the browser in order to log in to their router

    - NOTE: The router login is often set to the default (often "admin" and "password") but may have been changed by the customer

  - NOTE: Changing DHCP, WET, and Bridging settings will vary by router model but are often found in the "Advanced" section of the router settings menu

## 1333

- Meaning

  - Looking for Hopper

- Cause

  - Joey is not linked to Hopper due to a connection issue or authorization issue

- Recommended Resolution

  - Verify the Joey is active on the customer account

  - Perform a front panel reset on the Joey and Hopper

  - Ensure the Joey is linked to the Hopper in SETTINGS > WHOLE HOME

  - Inspect the signal path starting at the Joey and working back toward the Hub/Node

## 1503

- Meaning

  - A problem has been detected with the Super Joey

- Cause

  - Super Joey and Hopper have different satellite matrices, Super Joey was turned off during Check Switch Test, Cable to Super Joey is damaged or not correctly connected

- Recommended Resolution

  - Restart the Hopper and Super Joey

  - Re-run the check switch on the Hopper

  - Check the signal at the Super Joey with the SAT meter

  - Check the cable configuration making sure it's all hooked up correctly

    - Cable to LNB is connected to the right port

    - Port 1 is on port 1 and port 2 is on port 2

  - Make sure the cable and connectors are approved and in good shape

## 1553

- Meaning

  - Wireless Joey disconnected from Access Point

- Cause

  - This issue occurs when the Wireless Joey is not connected to the Access Point (AP)

- Recommended Resolution

  - Unplug AP and Wireless Joey then power on to re-establish connections

    - Follow on screen instructions to reconnect the Wireless Joey to the AP

  - Reposition AP and Wireless Joey closer together and away from other electronic devices such as routers

## 1592

- Meaning

  - Snap component failing

  - Hopper USB port failing

- Cause

  - This issue occurs when the Hopper Snap device is not connected/working or the USB port is not functioning correctly

- Recommended Resolution

  - Replace the Snap, if the error persists replace the receiver

## 2501

- Meaning

  - HDMI Connection No Longer Working Properly

  - This screen will appear on the Hopper and indicates that you need to move the HDMI connection from the Hopper to the Hopper Plus

- If it does not show after 5 minutes

  - Move HDMI to Hopper Plus, it may be on Android Recovery Screen and will need to be replaced

  - If not on Android Recovery, reboot Hopper and Hopper Plus

    - Note: When you reboot the Hopper, the Hopper Plus will automatically reboot

- Recommended Resolution

  - In order to exit Hopper Plus mode and the 2501 screen, hold the Black Power Button (not Red Reset) on the front panel until the Hopper reboots

    - It should not return to 2501 when following this process

## 2505

- Meaning

  - Caused by either connectivity or activation issues

- Recommended Resolution

  - Check if activated and numbers were entered correctly in OFS

  - Check USB cable and connection

  - Reboot Hopper Plus

  - If the Hopper is connected to Wi-Fi and Ethernet, remove one method of connectivity after taking out of Hopper Plus mode. More information in Receiver Broadband Connectivity

  - If these steps do not resolve, red button reset on Hopper

## 3000

- Recommended Resolution

  - The USB process must be used for ALL new connect installations

  - Receivers are not targeted for IP downloads for new installs (NC)

    - On upgrade work orders, the Download ATV Files button may be used

## 9999

- Meaning

  - Refresh In Progress

- Cause

  - The User Interface detected that a refresh was needed.

- Recommended Resolution

  - Please wait for the receiver to complete the Refresh

Common Issues

## Android Recovery Screen

- If this appears on a Hopper Plus or Joey 4, it will need to be replaced

## App Issues, Joeys Unlinking, Pixelating/Freezing Video

- The customer's ISP-provided modem/router may interfere with the Hopper's network connection or MoCA network and cause these symptoms:

  - Pixelating

  - Freezing video

  - Hopper Plus/Joey 4 Unlinking

  - Apps not loading or streaming interrupted

- Verify there is only a single broadband connection

  - If connected to both Wi-Fi and Ethernet, issues may occur with Joeys linking and apps may not work

  - Use the steps in the Dual Hopper Broadband Connections page to disconnect Wi-Fi when necessary

- If Voice Search, Streaming Apps, or On Demand is not working, try these steps:

  - Voice search or apps are not working:

    - Reset router and Hopper/Hopper Plus

      - If this does not resolve, the customer's router may be interfering with the connection and a HIC or 2nd Subnet may be needed

  - If the previous steps did not work, Install a HIC or Create a 2nd Subnet:

    - Install a HIC

    - If installing a HIC is not possible, create a 2nd subnet by connecting a second router:

      - Clear the existing Wi-Fi network from settings:

        - Diagnostics > Network > Internet Settings

        - Press INFO > RECALL on the remote

      - Once the second router is connected to their main router, connect the Hopper to the new router

## Black Screen - Guide & Menus Accessible

- Meaning

  - Receiver is in a fail state

- Cause

  - Receiver has temporarily malfunctioned

- Recommended Resolution

  - Check the cable and the software to make sure that it is current

  - If so, reboot receiver using front panel Reset button or by unplugging power cord

  - For Joeys, verify that the Joey is linked to the Hopper

  - If this does not solve the issue, check the HDMI

## Black Screen - No Guide

- Meaning

  - Guide is incomplete or not showing

- Cause

  - HDMI issue

  - No signal

  - Customer hit cancel download on VIP receiver

  - Customer did not let download happen

  - DVR was set causing download to skip

- Recommended Resolution

  - Run check switch, let guide download, check software version

## Channel is Missing in Guide

- Meaning

  - One or more channels are not appearing in the Electronic Program Guide

- Cause

  - Receiver has partial or complete signal loss or programming is not authorized

- Recommended Resolution

  - Verify programming is active on account

  - Additionally, verify that the favorites tab is not selected as this will only show the selected channels (not all of the subscribed channels)

  - Perform a check-switch test at the receiver location

  - Using a satellite meter, check signal at the receiver location and work back toward the dish antenna

  - Check signal at each connection point

  - Examine coax cable, connectors and components for proper installation and approved parts

## Factory Test Mode

- Meaning

  - Receiver in factory test mode

- Cause

  - Receiver was not set to normal operating mode when it left factory

- Recommended Resolution

  - Reset to factory defaults in DIAGNOSTICS > TOOLS > RESET to factory defaults

## Forced Download Failed

- If any of the software versions say “Failed” during the Force Download: Restart Hopper and then restart download

## Google Sign-in Issues

- Meaning

  - Customer is unable to sign into their Google account on the Hopper or Joey

- Recommended Resolution

  - Wait 10 minutes before rebooting the ATV device

## Hopper Plus or Joey 4 not Downloading

- Ready to install Hopper Plus and Joeys Checklist:

  - 1. All equipment activated?

  - 2. Hopper has the Android software available?

    - Hopper Plus Software Update Process

    - To confirm the latest ATV software, visit the USB Software Updates page

  - 3. Hopper is on Live TV?

  - 4. Hopper is IP-connected and bridging enabled?

- Verify there is only a single broadband connection

  - If connected to both Wi-Fi and Ethernet, issues may occur with Joeys linking and apps may not work

  - Use the steps in the Hopper Broadband Connections

    - 1. On the Hopper, access Diagnostics > Settings > Network > Internet Settings

    - 2. Press INFO > RECALL

    - 3. Select Clear Settings to disable the wireless connection to the network

- After the above steps are verified but the products are still not downloading, reboot the Hopper Plus/Joey that is having the issues WITHOUT rebooting the Hopper.

  - After these steps, the Hopper Plus may stall at 50% but should continue within 10 minutes

    - Rebooting the Hopper while the Hopper Plus is flashing software may cause failure

  - If the issue persists, then reboot the Hopper

- WJ4:

  - Connect an ethernet cable between Hopper and Joey

    - Once WJ4 is connected via ethernet and linked to the Hopper, continue the installation wizard steps

  - Is the orange light on the WJAP staying illuminated?

    - If not, reboot the WJAP

  - If the WJ4 is downloaded but is repeatedly dropping from the WJAP, the customer's router may be interfering with the connection and a HIC or 2nd Subnet may be needed

- If the previous steps have been followed and issues persist, temporarily disconnect the Hopper from broadband

  - Attempt the software update process on the Hopper Plus/Joey 4 again

  - Once the update is complete, reconnect the Hopper to broadband

## "Invalid" Software during USB Update

- Meaning

  - Invalid error message displays when preforming USB software update

- Cause

  - This is expected and does not cause issues with installation

- Recommended Resolution

  - Current factory software does not recognize the latest software versions on the USB drive

  - Once newer versions of factory software are released, this will be fixed

## Joey Will Not Activate

- If the Joey shows not activated in the Install Summary and RCAID has been verified active with dash, exit the install wizard

- To exit Install Wizard: Skip forward > recall > skip forward

  - If the Joey is still not showing activated once out of the Install Wizard, verify it is linked in Whole Home

## Missing Guide Info

- Meaning

  - Guide is incomplete or not showing

- Cause

  - No signal

  - Customer hit cancel download on VIP receiver

  - Customer did not let download happen

  - DVR was set causing download to skip

- Recommended Resolution

  - Make sure all channels are selected and not on favorite list

## No Audio

- Meaning

  - Video with no Audio

- Cause

  - HDMI

  - Faulty TV speakers

  - Check Dolby audio settings on receiver if on turn off or vice versa

- Recommended Resolution

  - Check HDMI, make sure Soundbar is turned on

  - Reset box, try a different channel

  - select different source and see if audio comes out to eliminate TV being the main issue

  - If other source works replace receiver

## Not Authorized

- Meaning

  - Not authorized

- Cause

  - Receiver is not active on the account or the customer has unplugged their receiver

- Recommended Resolution

  - Verify receiver is active in OFSC or by calling and confirming with a call center agent, send a "HIT" from OFSC

## Netflix

- Meaning

  - Netflix buffers/will not load

- Cause

  - Video is Frozen - Buffering

- Recommended Resolution

  - Verify Hopper/Wally is connected to internet, Run speed check Netflix needs 3Mbps for SD(5Mbps for HD and 25 for 4K). Reset Netflix App. Install a Hopper internet connecter.

## "Playback Failed"

- Meaning

  - Screen displays "Playback Failed"

- Cause

  - This issue has commonly been observed when the device is connected to the HDMI ARC port on the TV

- Recommended Resolution

  - Until a fix for this is identified, it is recommended to avoid connecting either a Hopper Plus, Joey 4, or Wireless Joey 4 to the ARC port

  - Note: Sound bars can still use the ARC port for audio

  - If the HDMI is not connected to the ARC port and the issue persists, inform the customer this is a known issue, a software fix is expected in the near future, and a reboot will temporarily resolve the issue

## Receiver Reboots/Turns Off

- Meaning

  - Receiver reboots or goes into standby

- Cause

  - Hard Drive failure, electrical problems, update or no activity within 4 hours causing stand by

- Recommended Resolution

  - Run STBH checking for hardware failure

  - Check cable is not damaged and braid is not touching stinger any short in the cable can cause a reboot

  - Ask customer when the reboot happens, if reboot happens after an update this is normal

  - Go into settings and change update time

## Receiver Wont Turn on

- Meaning

  - No power at receiver

- Cause

  - Hard Drive failure, electrical problems

- Recommended Resolution

  - Check electrical with receptacle tester making sure power is on

  - Plug power into another outlet. Verify outlet is not connected to wall switch. Replace receiver

## Remote Not Pairing to Joey 4

- Hold button on top of box for 10 solid seconds > Press SAT

- If this does not work, reset the receiver and try again

- Try a different remote

  - Note: You may need to replace the receiver if this does not work after multiple attempts

## Trick Modes (Skip Forward/Back, Rewind/Fast Forward)

- Customers may experience inconsistent responses when using the trick mode buttons on the remote (fast forward/rewind and skip forward/skip back)

  - Examples:

    - Pressing Skip Forward once or multiple times may not advance in the correct increments of 30 seconds

    - Fast forwarding may jump around and not progress smoothly through the show

## Software Version P056, P057, P058, P060 in Diagnostics

- The receiver did not go through the Install Wizard properly

- A Factory Reset will need to be initiated:

  - SETTINGS > DEVICE PREFERENCES > ABOUT > FACTORY RESET